Computer Organization and Architecture — CSE23CT201

Unit 1 · Basic Structure of Computers — Question Bank

Five-mark questions with internal split-up · Bloom's level tagged · Total 35 marks

Q1 · Chapter 1 — Basic Structure of Computers   [5 marks]

a)  List the five functional units of a computer.   (2 marks, L1 – Remember)

b)  Explain the roles of the ALU and the Control unit.   (2 marks, L2 – Understand)

c)  In the role-play program, why does the result come out wrong with no coordinator — what does the clock guarantee?   (1 mark, L4 – Analyze)

Q2 · Chapter 2 — Basic Operational Concepts   [5 marks]

a)  Define the stored-program concept.   (1 mark, L1 – Remember)

b)  State the roles of PC, IR, MAR and MDR.   (2 marks, L2 – Understand)

c)  For C = A + B (Load R2,A · Load R3,B · Add R4,R2,R3 · Store R4,C), write the RTN of each instruction.   (2 marks, L3 – Apply)

Q3 · Chapter 3 — Bus Structures   [5 marks]

a)  What is a bus? Name the three buses by function.   (2 marks, L1/L2)

b)  Differentiate single-bus and multiple-bus organization.   (2 marks, L4 – Analyze)

c)  Why does a single bus become a bottleneck as the processor speeds up?   (1 mark, L4 – Analyze)

Q4 · Chapter 4 — Memory Locations & Addresses   [5 marks]

a)  Define byte-addressability.   (1 mark, L1 – Remember)

b)  Differentiate big-endian and little-endian by storing the 32-bit value 0x11223344 at address 1000.   (2 marks, L3 – Apply)

c)  Words are 4 bytes and must be aligned. Starting at address 1000, give the next two word addresses, and state whether address 1002 could start a word.   (2 marks, L3/L4)

Q5 · Chapter 5 — Memory Operations & Instructions   [5 marks]

a)  Write the RTN for a memory Read and a Write.   (2 marks, L2 – Understand)

b)  Classify instructions by number of operands.   (1 mark, L2 – Understand)

c)  Differentiate RISC and CISC; state which permits Add B, A and why.   (2 marks, L4 – Analyze)

Q6 · Chapter 6 — Addressing Modes   [5 marks]

a)  Give the notation and effective address for any five addressing modes.   (3 marks, L2 – Understand)

b)  Given R5 = 1000 and memory[1020] = 42, compute the EA and operand for Load R2, 20(R5).   (1 mark, L3 – Apply)

c)  Which mode suits linked-list traversal versus array scanning, and why (compare memory accesses)?   (1 mark, L4 – Analyze)

Q7 · Chapter 7 — Instruction Sequencing   [5 marks]

a)  Differentiate straight-line sequencing and branching.   (2 marks, L2 – Understand)

b)  Differentiate unconditional and conditional branch, with an example each.   (2 marks, L2/L3)

c)  A loop uses Branch>0 LOOP. The branch is at address 116 and LOOP is at 100 (4-byte instructions). What offset is stored, and how is the target computed?   (1 mark, L4 – Analyze)