Unit 1 — Basic Structure of Computers

CSE23CT201 · Computer Organization & Architecture · CO1 — Topic-wise exam question bank (what to write in the exam)

# Chapter 1 — The Five Functional Units

## The five functional units (definition to reproduce)

Input — accepts coded information from outside (keyboard, mouse, sensors, network).

Memory — stores the programs and data.

ALU (Arithmetic & Logic Unit) — performs the arithmetic and logic operations.

Output — sends the results to the outside world.

Control unit — coordinates all the units with timing signals.

ALU + Control + registers together form the PROCESSOR.

Five functional units — data path (solid), control signals (dashed).

## Control unit

Generates two kinds of signal: clock (timing) signals = WHEN each step happens; control signals = WHAT to do on that tick.

Reads and decodes the instruction in the IR and drives every other unit at the right instant.

## Clock

A steady square-wave pulse — the computer’s heartbeat, sent to every unit.

One pulse = one step (one microoperation).

Ticks per second = clock rate in hertz (e.g. 3 GHz = 3,000,000,000 ticks/second).

## Memory hierarchy (clean 5-mark answer)

Primary / Main memory — fast semiconductor RAM; holds the running program and data; volatile.

Cache — small, very fast RAM near the processor; holds frequently-used instructions/data.

Secondary storage — disk, SSD, flash; large, cheap, non-volatile permanent storage.

Speed order (fastest first): Register → Cache → Main memory → Disk.

# Chapter 2 — Basic Operational Concepts

## Stored-program concept

Both the program instructions and the data are stored together in the same memory, each instruction at its own address (100, 104, 108 …).

The processor fetches instructions one at a time and executes them.

## Processor registers (write these four one-liners — 4 marks)

PC (Program Counter) — holds the address of the NEXT instruction to be fetched.

IR (Instruction Register) — holds the instruction CURRENTLY being executed.

MAR (Memory Address Register) — holds the address of the memory location to be accessed.

MDR (Memory Data Register) — holds the data being written to, or just read from, memory.

## RTN (Register Transfer Notation)

[A] means the CONTENTS of location A; ← means “copied into”.

The source is unchanged; the destination is overwritten.

## Demo program: C = A + B (textbook load/store convention)

| Addr | Instruction | RTN |
| --- | --- | --- |
| 100 | Load R2, A | R2 ← [A] |
| 104 | Load R3, B | R3 ← [B] |
| 108 | Add R4, R2, R3 | R4 ← [R2] + [R3] |
| 112 | Store R4, C | M[C] ← [R4] |

## Instruction cycle (classic 5-mark answer)

Phase 1 — Fetch & Decode: PC → MAR → memory; the instruction is read into MDR, then into IR; the control unit decodes the IR.

Phase 2 — Execute: the operand address goes to MAR, the operand is read into MDR, the ALU acts, the result is stored, and the PC is incremented.

Written as three steps: fetch · decode · execute.

# Chapter 3 — Bus Structures

## Bus — definition (reliable 5-marker)

A bus is a shared set of wires that transfers data and control information between the functional units.

Only one source may drive it at a time (built with multiplexers or tri-state buffers).

A bus = three groups of lines: address, data, control.

## The three line-groups

Address bus — carries the address to access; k lines → 2^k locations; usually one-way (processor → memory).

Data bus — carries the data / instruction word; both ways.

Control bus — carries Read/Write, timing and status signals.

## Single vs multiple bus

Single bus — one shared path; cheap and simple; only one transfer at a time (bottleneck).

Multiple / two-bus — parallel paths; faster; costlier (more wiring).

System bus — the main highway tying processor (via cache), main memory and I/O; bundles address + data + control lines.

# Chapter 4 — Memory Locations & Addresses

## Bit, byte, word

Bit — a single 0 or 1 (smallest unit).

Byte — a group of 8 bits (holds one character).

Word — the n-bit unit the processor handles at once; word length typically 16 / 32 / 64 bits.

## Addressing facts (examiners love these)

Byte-addressable — every 8-bit byte has a distinct address (0, 1, 2, …).

A k-bit address → 2^k addressable locations (2^24 = 16M; 2^32 = 4G).

Aligned word — begins at an address that is a multiple of the word size in bytes (32-bit word: 0, 4, 8 … aligned; 6 is not).

## Endianness

Big-endian — most-significant byte at the lowest address.

Little-endian — least-significant byte at the lowest address.

Storing 0x11223344 at address 1000 — big-endian vs little-endian.

# Chapter 5 — Memory Operations & Instructions

## The two memory operations (in exam words)

Read — the processor sends an address; memory returns a COPY of the contents; source unchanged. RTN: DR ← [LOC].

Write — the processor sends an address and data; memory stores it, overwriting the old value. RTN: M[LOC] ← R1.

Read and Write — address through MAR, data through MDR.

## Instruction basics

Instruction = opcode (the operation) + operand(s) / address (what it acts on).

Assembler — translates symbolic assembly (mnemonics) into the binary object program.

Instruction formats — memory-reference, register-reference, and input–output.

## Classify by number of operands (same job C = A + B)

| Type | Example | What is implied |
| --- | --- | --- |
| 3-address | Add C, A, B | nothing — all three named |
| 2-address | Add C, A | destination doubles as a source |
| 1-address | Add B | the accumulator: AC ← [AC] + [B] |
| 0-address | Add | the top two stack entries |

## RISC vs CISC

|  | RISC | CISC |
| --- | --- | --- |
| Instruction set | few & simple (often < 100) | many (100s–1000s), memory-to-memory |
| Execution | one cycle → easy to pipeline | variable length → multi-cycle |
| Memory access | load/store only (Load/Store touch memory) | arithmetic may act directly on memory |
| Burden on | the compiler (software) | the hardware |
| Examples | ARM, RISC-V | x86 (Intel/AMD) |

# Chapter 6 — Addressing Modes

## Addressing modes with effective address (guaranteed question)

| Mode | Notation | Effective address |
| --- | --- | --- |
| Immediate | #Value | operand is in the instruction |
| Register | Ri | operand is in Ri |
| Absolute / Direct | LOC | EA = LOC |
| Indirect | (Ri) | EA = [Ri] |
| Index | X(Ri) | EA = X + [Ri] |
| Relative | X(PC) | EA = X + [PC] |
| Autoincrement | (Ri)+ | EA = [Ri], then Ri ← Ri + 1 |
| Autodecrement | −(Ri) | Ri ← Ri − 1, then EA = [Ri] |

## Worked examples to quote

Immediate: Move R2, #5 → R2 ← 5.

Direct: Load R2, A → R2 ← [A].

Indirect: Load R2, (R5) → R2 ← [[R5]] (two memory accesses).

Index: Load R2, 20(R5) → EA = 20 + [R5]; if R5 = 1000 then 20(R5) reaches address 1020.

# Chapter 7 — Instruction Sequencing

## The exam essentials

Straight-line sequencing — the PC is auto-incremented after each fetch, so instructions execute in increasing address order.

Branching — a branch loads a NEW address into the PC instead of PC+4. Two kinds: unconditional and conditional.

Unconditional branch (jump) — always taken; PC ← target every time (e.g. loop back).

Conditional branch — taken only if a tested condition is true. The preceding Compare/arithmetic sets the condition-code flags (Z, N, C, V), which the branch reads (e.g. Branch>0).

Branch target = the destination address; offset = target − (PC of next instruction); relative EA = PC + offset.