Computer Organization and Architecture — CSE23CT201

Unit 2 · Register Transfer, Microoperations & Control — Question Bank

Five-mark questions with internal split-up · Bloom's level tagged · Core 25 + Extended 15 = 40 marks

Section A · Core — one question per chapter   [5 questions × 5 = 25 marks]

Q1 · Chapter 1 — Register Transfers & Microoperations   [5 marks]

a)  Define a microoperation, and differentiate an instruction from a microoperation.   (2 marks, L1/L2)

b)  For the conditional transfer P : R2 ← R1, write it in register transfer language and explain the role of the control function P.   (2 marks, L2/L3)

c)  Two registers must place their contents on one common bus. Explain how tri-state buffers avoid a conflict, and what goes wrong if two are enabled at once.   (1 mark, L4 – Analyze)

Q2 · Chapter 2 — Arithmetic, Logic & Shift Microoperations   [5 marks]

a)  Show how A − B is performed with a binary adder and 2's complement (state the role of the carry-in Cin); explain why no separate subtractor is needed.   (2 marks, L3 – Apply)

b)  A single arithmetic circuit performs add, subtract, increment and decrement. Explain how the input multiplexer's select lines and the carry-in choose the operation.   (2 marks, L2/L4)

c)  Using a mask, show how a bitwise AND clears selected bits while a bitwise OR sets selected bits.   (1 mark, L3 – Apply)

Q3 · Chapter 3 — Execution of Complete Instructions   [5 marks]

a)  List the five steps in the execution of a complete instruction, and state what the fetch step does.   (2 marks, L1/L2)

b)  Trace the instruction Load R2, A through the single-bus datapath, writing the RTN performed at each step.   (2 marks, L3 – Apply)

c)  Why is the RY multiplexer needed at the register-file input — what two sources can supply the value written back?   (1 mark, L4 – Analyze)

Q4 · Chapter 4 — Bus Organization   [5 marks]

a)  Differentiate single-bus and three-bus (multiple-bus) organization of the datapath.   (2 marks, L4 – Analyze)

b)  For Add R4, R2, R3, state how many steps are needed on a single-bus datapath versus a three-bus datapath, and explain the difference.   (2 marks, L3/L4)

c)  Which organization suits a low-cost design and which suits a high-performance (RISC) design — justify.   (1 mark, L4 – Analyze)

Q5 · Chapter 5 — Control Unit   [5 marks]

a)  Name the three components of a hardwired control unit (instruction decoder, step counter, control-signal generator) and state the role of each.   (2 marks, L1/L2)

b)  Write a control signal as a Boolean equation — e.g. Read = T1 + T4·LOAD — and explain what each term contributes.   (2 marks, L2/L3)

c)  Differentiate hardwired and microprogrammed control, and state which suits RISC and which suits CISC, with reasons.   (1 mark, L4 – Analyze)

Section B · Extended coverage — remaining topics   [3 questions × 5 = 15 marks]

Q6 · Chapters 1–2 — Memory Transfers, Logic & Masking   [5 marks]

a)  Write the RTN for a memory Read and a memory Write, showing that all data passes through the MDR and the address through the MAR.   (2 marks, L2 – Understand)

b)  Given a 16-bit register, use mask-and-shift microoperations to extract the field in bits [7:4], then insert a new 4-bit value into that field.   (2 marks, L3 – Apply)

c)  Why are there exactly sixteen logic microoperations, and how do four gates (AND, OR, XOR, NOT) generate all of them?   (1 mark, L4 – Analyze)

Q7 · Chapter 2 — Shift, Multiply & Divide   [5 marks]

a)  Show that a left shift multiplies by 2 and a right shift divides by 2 (with an example), and explain why arithmetic shift-right (ashr) must preserve the sign bit.   (2 marks, L3/L4)

b)  Multiply 5 × 3 using the shift-and-add algorithm, showing the partial products.   (2 marks, L3 – Apply)

c)  Explain how division mirrors multiplication as a shift-and-subtract process.   (1 mark, L2 – Understand)

Q8 · Chapters 3 & 5 — Addressing & Microprogramming   [5 marks]

a)  For the indexed load Load R2, 20(R5) with R5 = 1000 and memory[1020] = 42, compute the effective address and the operand loaded.   (2 marks, L3 – Apply)

b)  Describe the anatomy of a control word (microinstruction): what each bit represents, and the purpose of the End bit.   (2 marks, L2 – Understand)

c)  List the four ways the μPC selects the next microinstruction (the sequencing methods).   (1 mark, L1 – Remember)

Bloom's Taxonomy — Coverage Map

Marks are attributed to the dominant cognitive level of each sub-part (split parts shared evenly). The paper is weighted toward Apply and Understand, with Analyze for higher-order discrimination.

| Level | Cognitive skill | Typical verbs used here | Marks | Share |
| --- | --- | --- | --- | --- |
| L1 | Remember | define, list, name, state | 4 | 10% |
| L2 | Understand | explain, differentiate, describe | 11 | 27.5% |
| L3 | Apply | compute, trace, show, multiply | 15 | 37.5% |
| L4 | Analyze | compare, justify, why | 10 | 25% |
|  |  | Total | 40 | 100% |