Unit 2 — Register Transfer, Microoperations & Control

CSE23CT201 · Computer Organization & Architecture · CO2 — Topic-wise exam question bank (what to write in the exam)

# Chapter 1 — Register Transfers & Microoperations

## Key definitions (high-frequency answers)

Microoperation — an elementary operation on register data carried out in one clock pulse (load, clear, count, shift, add).

Instruction vs microoperation — one instruction is carried out as a SEQUENCE of microoperations.

RTL: R2 ← R1 — source unchanged, destination overwritten. Symbols: letters = a register, ( ) = part of a register, ← = transfer, comma = two transfers in the same clock.

Control function: P : R2 ← R1 — the transfer occurs only when P = 1.

## Common bus

Built from multiplexers, OR from tri-state buffers + a decoder.

Only one source may drive the bus at a time; enabling two at once causes bus contention (an invalid value).

## Memory transfers (all data passes through MDR)

Address in MAR, data in MDR; there is no direct register↔memory path.

Read (fetch a word): MDR ← M[MAR].

Write (store a word): MDR ← R1, then M[MAR] ← MDR.

# Chapter 2 — Arithmetic, Logic & Shift Microoperations

## Arithmetic microoperations

Operations: add, subtract (via 2’s complement), increment, decrement.

Subtraction: A − B = A + (2’s complement of B) = A + B′ + 1.

## Adder–subtractor (one circuit, two jobs)

Each B bit passes through an XOR gate controlled by a mode line M; the carry-in Cin = M.

M = 0 → B unchanged, Cin = 0 → A + B (ADD).

M = 1 → B flipped, Cin = 1 → A + B′ + 1 = A − B (SUBTRACT). Cin = 1 supplies the “+1”.

Example — 5 − 3: 3 = 0011 → flip → 1100 → +1 → 1101; 0101 + 1101 = 1 0010; drop the carry → 0010 = 2.

## Logic (bitwise) microoperations

AND — mask / clear a bit; OR — selective-set a bit; XOR — toggle; NOT — complement.

There are 16 logic microoperations, all built from 4 basic ones: AND, OR, XOR, NOT.

Clear a flag bit (lets clear the third bit) with AND:

Flags-register contains 1011

Now let’s AND 1101(0s only where you want to clear) with the reg to clear the third bit

1001 is the result of that AND (that bit → 0, others unchanged).

Similarly, Set with OR: flags 1001 OR 0100 = 1101 (that bit → 1, others unchanged).

Extract a field: word AND 0000 1111 keeps the low nibble. Insert a field: AND to clear the field, then OR the new bits in.

## Shift microoperations

Logical shift — 0 fills the vacated end.

Arithmetic shift — the sign bit is preserved on shift-right (ashr), so dividing a negative number by 2 stays negative.

Circular shift — the bit wraps around.

Left shift = ×2, right shift = ÷2; shift by k = ×2^k or ÷2^k.

## Multiply & divide

Multiply = shift-and-add: for each 1 in the multiplier, add a shifted copy of the multiplicand.

An n-bit multiplier needs at most n shifts + n adds.

Divide = shift-and-subtract (the mirror of multiply).

## ALU

The ALU = arithmetic unit + logic unit + shift unit, the operation chosen by select lines.

# Chapter 3 — Execution of Complete Instructions

## The datapath

Register file — two read ports (A, B) and one write port (C).

Interstage registers — RA, RB (operands), RZ (ALU result), RM (memory data out), RY (data written back).

A multiplexer (MuxY) chooses the ALU result (for arithmetic) or the memory data (for a load); RY holds the value for write-back.

Processor datapath — register file → RA/RB → ALU → RZ → RY → back to port C; RM to memory.

## The five-step sequence

1 Fetch — read the instruction at [PC]; increment the PC.

2 Decode — work out the operation; read the source registers.

3 Compute — the ALU performs the operation or computes an address.

4 Memory — read/write data (only for Load / Store).

5 Write-back — the result returns to the destination register.

## Trace: Load R2, A (absolute)

| Step | Register transfers |
| --- | --- |
| 1 Fetch | MAR ← [PC]; MDR ← M[MAR]; IR ← [MDR]; PC ← PC + 4 |
| 2 Decode | opcode = Load; dest = R2 |
| 3 Compute | — no ALU work (A is already the address) |
| 4 Memory | MAR ← A; MDR ← M[MAR]; RY ← [MDR] |
| 5 Write-back | R2 ← [RY] |

## Effective address (indexed load)

Load R2, 20(R5): effective address = [R5] + 20; the ALU adds base + offset in the Compute step.

If R5 = 1000, the effective address is 1020.

# Chapter 4 — Bus Organization

## Single-bus vs three-bus

Single-bus — one shared internal path; one transfer per clock; cheap but slow (bottleneck). Memory is reached only through MAR (address) and MDR (data).

Three-bus (multiple-bus) — Bus A and Bus B carry both operands to the ALU at once; Bus C returns the result — the whole operation in one step.

Add R4, R2, R3: single-bus ≈ 3 steps; three-bus = 1 step. More buses = fewer clocks, more wiring.

Single-bus (one shared path) vs three-bus (A, B in — C out) datapath.

## Who builds which

Single/multi-bus datapath — micro-programmed CISC (8086, 68000, VAX) and simple microcontrollers (8051-class, IoT nodes); chosen when cost/area/power matter.

Five-beat pipeline — RISC machines chop the same datapath into five fixed stages and overlap instructions for throughput.

# Chapter 5 — Control Unit

## What the control unit does

Turns (IR + step counter + condition flags Z,N,C,V) into control signals, every clock.

Drives the control points: register loads (MARin, MDRin …), bus enables (PCout …), the ALU function, memory Read / Write.

Two ways to build it: hardwired or microprogrammed.

## Hardwired control

Three parts: instruction decoder + step counter (T1–T5) + control-signal generator (combinational logic).

Each control signal is a Boolean equation, e.g. Read = T1 + T4·LOAD, and MARin = T1 + T4·(LOAD+STORE).

Fast but rigid (changing an instruction means redesigning the logic) → suits RISC.

## Microprogrammed control

Control signals are stored as microinstructions in a control store; a μPC / CAR and an address generator sequence them.

Microinstruction = control word (the signal bits) + sequencing word (the next address); the End bit ends a microroutine.

Flexible but slower (a control-memory read each step) → suits CISC.

Sequencing (4 ways the μPC picks the next microinstruction): increment (in-line), branch / conditional branch, opcode map, End.

## Hardwired vs microprogrammed (exam table)

| Property | Hardwired | Microprogrammed |
| --- | --- | --- |
| Decision lives in | fixed logic gates | a control store (memory) |
| Speed | faster — one gate delay | slower — a memory read per step |
| Flexibility | rigid — rewire to change | flexible — rewrite microcode |
| Complex instruction sets | hard (huge logic) | easy (just more microcode) |
| Best fit | RISC (simple, regular) | CISC (rich, complex) |